Roadworks: Engineering Drawing Checklist

Privacy Statement: The information collected on this form will be used by Brisbane City Council – Development Assessment for the purpose of ensuring all key design criteria is provided as part of Operational Works & Compliance Assessment application submissions.

It is the responsibility of the consultant to ensure all information is provided in accordance with Brisbane City Council’s Codes and Policies to accommodate a streamlined approval process.

| Site Address: |  |
| --- | --- |
| High Level Application #: |  |
| Condition Reference #: |  |

| Description | Completed |  | Comments To be completed for all non-standard designs |
| --- | --- | --- | --- |
|  | YES | NO |  |
| Preliminary Requirements Refer to Section 1.5 > Chapter 1 Introduction > Infrastructure Design (ID) PSP > Schedule 6 PSP Note: Drafting must be of a standard that is acceptable for construction in civil engineering & architectural practice, in accordance with the requirements of AS 1100.101-1992 Technical drawing – General principles. |  |  |  |
| Have plans been signed & checked by RPEQ together with RPEQ number and date shown on title block? |  |  |  |
| Plans assessed against all town planning conditional requirements & approved plans: - ROL/MCU conditions - Any road dedications / corner truncations |  |  |  |
| Leading Drawing must contain: - Council reference number; - Site Address (consistent with application); - Locality plan (showing stage boundaries if applicable); - Drawing index; - List of all Council standard drawings used; - List of all consultant standard drawings used (copies to be attached); - Full Legend; - Existing & Proposed Contours |  |  |  |
| Scales: Recommended Scales are 1:1, 1:2, 1:5 (multiplied by integral powers of 10) - Plans – 1:1000 or 1:500 (roof-water reticulation layout plans should be drawn in the 1:500 scale); - longitudinal sections – (i) horizontal 1:1000 and vertical 1:100; or (ii) horizontal 1:500 and vertical 1:50. - Intersection details – 1:200, 1:100 or 1:250; - cross-sections – 1:100; - Engineering details – 1:20 or 1:10. - A north point and legend is to be shown on all drawings. |  |  |  |
| Dimensions: - All dimensions in metric units. - Linear dimensions in metres. - Excl. detailed drawings (maintenance holes/ kerb & channel) – millimetres. - Reduced levels of roadworks & stormwater drainage expressed & rounded to 3 decimal places (0.001m) - Chainages to 3 decimal places (0.001m) - Road cross-sections provided at 20m intervals (further subdivision of 10m to 5m where necessary at horizontal or vertical clearances). - Road & Pipe grades must be show to 3 significant figures (e.g. 2.635%). |  |  |  |

| General Checks |  |  |  |
| --- | --- | --- | --- |
| Ecology Triggers |  |  |  |
| - Are there any Ecology conditions (e.g. vegetation management plan, landscape, ecology or NALL) which may affect the proposed designs. - If YES: is the required VMP OpWorks approved? - If NO: can the application be approved with the generic stamp and amendments in red (will required advice from BCC Ecologist). |  |  |  |
| - Is the site on the contaminated land register or Environmental Land Register? |  |  |  |
| - Identify areas subject to Vegetation protection or NALL on plans. |  |  |  |
| Flooding & Waterway implications |  |  |  |
| - Is the site affected by flooding? - If YES; details of minimum habitable floor levels are to be clearly identified on the layout plans where allotments are to be filled to provide flood immunity (refer to Flood Overlay Code & Planning Approval conditions). |  |  |  |
| - Effect on neighbouring properties examined (ponding) |  |  |  |
| - Location of Waterway Corridors identified on plans (if applicable). |  |  |  |
| - Location of all flooding extents including drainage/overland flow easements identified. |  |  |  |
| Approved Functional Layout Plan (if applicable) |  |  |  |
| - Designs conform to previously approved Functional Traffic Layout plan. |  |  |  |
| Roadworks Layout Plan |  |  |  |
| Site Characteristics (Existing & Proposed) |  |  |  |
| - Surface Levels - Accesses - Services - Larger Trees - Guard Rails & Delineators - Retaining Walls - Combined Services - Road reserve boundaries & road identification - Contours (existing AND proposed) - Hazard signs locations - Subsoil drainage: both sides of road & island treatments - Future road extensions - Pavement tapers are to be a minimum of 1V:10H as a transition between the new and existing pavements of differing width |  |  |  |
| Pavement Treatments |  |  |  |
| - Asphalt (pavers not permited), • pattern complies with Guidelines • sub-grade treatment CTB or concrete |  |  |  |
| - Stencilled Concrete, • pattern complies with Guidelines • colour complies with Guidelines • sub-grade treatment shown • concrete pavement depth satisfactory • extra conduits provided. |  |  |  |
| - Street Print • Colour & pattern complies with Guidelines • 32mm thick AC specified for indentation |  |  |  |
| Pavement Widths |  |  |  |
| - Arterial – 24.5 (MINIMUM) - Suburban – 24.5 (MINIMUM) - District – 11m - Neighbourhood (non-bus) – 7.5m - Neighbourhood (Bus) – 11m - Local – 5.5m **Subject to requirements of conditions & special verge widths as per Streetscape & Bicycle network overlay codes |  |  |  |
| Kerb Profiles (Typical) Refer to BSD-2001 |  |  |  |
| - Laneways – N/A - Local Streets – Layback Type D - Neighbourhood Rd (50km/h) – Type E - Neighbourhood Rd (40km/h) Bus Route – Type E - Neighbourhood Rd (40km/h) – Type D - Major Rd – Type E - Medians & Traffic Islands – Mountable Type D Parks – Type E. |  |  |  |
| Intersection Layout |  |  |  |
| - Crossfalls • minimum 2 %, • maximum 5 %, - One-way crossfalls kerb drain, |  |  |  |
| - Drainage provided to sags |  |  |  |
| - Footpath & kerb ramp location satisfactory, |  |  |  |
| - Minimum radii provided, • residential R10 • industrial R15. |  |  |  |
| Cul-de sac Treatments (Refer BSD-3001 & 3002) |  |  |  |
| - All areas drain |  |  |  |
| - Layout satisfactory for current refuse collection vehicle turning radius (12.5m), |  |  |  |
| - Locations of refuse bin pads shown where req’d (left side collection only). |  |  |  |
| Geometry (refer to Ch.3 > IDPSP) |  |  |  |
| Road Longitudinal Grades |  |  |  |
| - MINIMUMS – 1% - MAXIMUMS - Arterial – 5% - Suburban – 6% - District – 6% - Neighbourhood (non-bus) – 16.7% - Neighbourhood (Bus) & Local – 10% - - Local Street – 16.7% |  |  |  |
| Vertical Geometry |  |  |  |
| - Access-ways / residential streets / collector roads conform to Queensland Streets. |  |  |  |
| - Sub-arterial/arterial complies with AUSTROAD |  |  |  |
| - Sag Radii maximum 1300m |  |  |  |
| Verges |  |  |  |
| Verge Widths (Refer to Road Hierarchy & Streetscape Hierarchy Overlays) |  |  |  |
| (Minimum widths as measured from property alignment to invert of kerb & channel) - Local & Neighbourhood Roads • Residential. 4.25m (typical) and • Rural 5.0m (min.) - District/Suburban & higher roads (without a bike lane) • 4.25m (min) • 5.95m (preferred) - District/Suburban & higher roads (with a bike lane) • Refer to Streetscape Hierarchy Overlay Code and/or Bicycle Network Overlay Code - Industrial Access roads 4.25m. |  |  |  |
| Verge Profile |  |  |  |
| - Crossfall to be 1 in 50 – full width - RURAL (where practical constraints limit std. profile) - • Section of verge min 2.5m wide @ ultimate level with max crossfall at 1 in 40 |  |  |  |
| Verge Longitudinal Grades |  |  |  |
| - Max 1 in 6¼ (ie. matches road long section). |  |  |  |
| Concrete Footpaths (Refer to BSD-5201-03) |  |  |  |
| - To comply with approved conditions, - Commercial development - 1.2m both sides, - Shop / shopping centre - full width or as approved by Urban Amenity. - Kerb ramps provided (BSD-5231) |  |  |  |
| Pathways & Bikeways |  |  |  |
| Access Pathways |  |  |  |
| - Refer to approved conditions & layout plan, |  |  |  |
| - 1.2m/1.8m wide concrete pathway shown, |  |  |  |
| - Vehicular access prevented, |  |  |  |
| - Kerb ramps provided at either end, |  |  |  |
| - Cross section suitable for overland flows, - Reserve contains overland flow width (where required) |  |  |  |
| - Minimum width reserve 3.0-4.5m |  |  |  |
| Bikeways |  |  |  |
| - Location & design acceptable to Council (as per conditions) • 2.5m wide- recreational • 3.0m wide (min.) – commuter |  |  |  |
| - Pavement details specified (Type A minimum) - complies with BCC standard drawings, |  |  |  |
| - Entrance and exit treatment at roads satisfactory (Speed reduction/safety rails), |  |  |  |
| - Longitudinal grades comply with AUSTROADS (minimum 0.5%, maximum 10%), |  |  |  |
| - Drainage shown (min. 1m clearance to path), |  |  |  |
| - Bike safe grates where req’d, |  |  |  |
| - Clearance to obstacles (minimum 300mm, desirable 500mm) as per AUSTROADS, |  |  |  |
| - Treatment at creek crossings shown, • hydraulics approved • structural certificate provided • deck levels at correct flood immunity • details of flood depth indicators shown • bikeway bridges min. 3.0m wide • safety berm to waterways & slopes shown. |  |  |  |
| Rear Lot Access Easements |  |  |  |
| Rear Lot Access |  |  |  |
| - Requirements as per Section 4.12 of TAPS PSP |  |  |  |
| - Standard footpath cross-section complies, |  |  |  |
| - Width of concrete d’way min. 2.5m full width, |  |  |  |
| - Service corridor min. 0.5m width, |  |  |  |
| - Width of access easement min. 3 5m, |  |  |  |
| - If trafficked by a refuse vehicle – min. 5.5m wide with pavement constructed to carry nominal loading (1.5 x 104ESA |  |  |  |
| - Kerb required as necessary |  |  |  |
| - Passing bays adequate for greater than 1 lot. |  |  |  |